# Canadian Advanced RPAS — Validated MCQ Bank II

Questions 101–350 • 250 additional multiple-choice questions • Current Canadian authority

# Scope

This second volume completes the remaining 250 questions across Air Law 1-3/1-4, Meteorology 4-3/4-4, Navigation 5-1/5-2, Flight Operations 6-1/6-2 and Radiotelephony 8-1/8-2. Questions are normalized to current Canadian requirements and are not attributed to any secondary practice source.

Authority hierarchy: Canadian Aviation Regulations (Justice Laws) for legal requirements; Transport Canada Standards 921/922 where applicable; TP 15263 Fourth Edition for Advanced knowledge objectives; TC AIM 2026-1 for operational procedures; and ISED RIC-21/ROC-A material for aeronautical radiotelephony.

# Authoritative reference index

Canadian Aviation Regulations (SOR/96-433)

TP 15263 — Advanced RPAS Knowledge Requirements

TC AIM — main page

TC AIM 2026-1 RAC — Rules of the Air and ATS

TC AIM 2026-1 MET — Meteorology

TC AIM 2026-1 COM — Communications

TC AIM 2026-1 AGA — Aerodromes

Standard 921 — Remotely Piloted Aircraft

Standard 922 — RPAS Safety Assurance

ISED — Restricted Operator Certificate Aeronautical (ROC-A)

ISED RIC-21 — ROC-A Study Guide

# Section 1 — Air Law, Air Traffic Rules and Procedures (1-3 / 1-4)

101. In Canadian Class D airspace, ATC normally provides separation between:

A. IFR aircraft; VFR traffic receives traffic information and conflict-resolution assistance as workload permits

B. All VFR aircraft

C. VFR aircraft only

D. No aircraft

Answer: A. Class D permits IFR and VFR. IFR traffic receives ATC separation; VFR receives information and assistance rather than full VFR-to-VFR separation.

Authority: TC AIM RAC - Airspace Classification

102. Which flight rules are permitted in Class C airspace?

A. IFR only

B. Both IFR and VFR, subject to the applicable ATC requirements

C. VFR only

D. Military only

Answer: B. Class C is controlled airspace used by both IFR and VFR traffic, with communications/clearance requirements.

Authority: TC AIM RAC - Class C

103. Before an Advanced RPAS operation enters controlled airspace, the RPAS pilot must obtain:

A. A municipal permit

B. Only an airport-manager email

C. Authorization from the air traffic services provider responsible for the airspace

D. A police clearance

Answer: C. CAR 901.71 requires authorization from the applicable ATS provider before an Advanced RPAS operation in controlled airspace.

Authority: CAR 901.71

104. If ATS requires two-way aeronautical radio communications for an RPAS operation, the person operating the radio should hold:

A. A marine ROC-M only

B. No radio qualification

C. A driver's licence

D. The appropriate aeronautical radio operator qualification, such as ROC-A

Answer: D. TP 15263 and ISED ROC-A material identify the aeronautical operator certificate as the applicable qualification for aeronautical-band voice radio operation.

Authority: TP 15263 Section 8 / ISED ROC-A

105. A VFR aircraft entering Class C airspace normally requires:

A. An ATC clearance

B. No communication

C. Only a listening watch

D. Only a flight service briefing

Answer: A. Class C VFR entry is subject to ATC clearance and communications requirements.

Authority: TC AIM RAC - Class C

106. Where should a pilot verify the dimensions and classification of Canadian controlled airspace?

A. An old road atlas

B. Current aeronautical charts and publications such as the VNC/VTA, DAH and CFS

C. A consumer weather app only

D. Manufacturer warranty documents

Answer: B. TP 15263 requires pilots to use current aeronautical charts and approved publications to identify airspace and aerodromes.

Authority: TP 15263 Section 5 - Maps and Charts

107. Before using an aerodrome for a crewed-aircraft takeoff or landing, the PIC must ensure:

A. The runway is paved

B. The aerodrome has a control tower

C. The aerodrome is suitable for the intended operation

D. The aerodrome is in Class C

Answer: C. CAR and TC AIM aerodrome procedures place responsibility on the PIC to determine suitability for the intended operation.

Authority: TC AIM RAC / AGA - Aerodrome Operations

108. In Class C airspace, ATC separation is provided between:

A. IFR aircraft only

B. VFR aircraft only

C. No aircraft

D. IFR aircraft and, as prescribed, between IFR and VFR aircraft

Answer: D. Class C provides a higher level of ATC service than Class D, including IFR/VFR separation as specified in the airspace rules.

Authority: TC AIM RAC - Airspace Classification

109. For an RPAS model subject to a safety-assurance declaration, information about hazards capable of causing serious injury belongs primarily in:

A. Safety/operating documentation supporting the declared operation

B. Advertising material

C. The pilot's logbook only

D. The airport CFS entry

Answer: A. Standard 922 requires technical safety assurance and supporting documentation appropriate to the declared operation.

Authority: Standard 922 - RPAS Safety Assurance

110. A manufacturer/declarant must retain verification evidence primarily to demonstrate:

A. Marketing performance

B. Compliance of the RPAS model with the applicable technical requirements

C. Pilot recency

D. Airport access permission

Answer: B. The declaration system depends on documented verification that the model meets the applicable Standard 922 requirements.

Authority: CAR 901.194-901.201 / Standard 922

111. A Class E control zone begins:

A. At FL180

B. At 12,500 ft AGL

C. At the surface

D. Only above 3,000 ft AGL

Answer: C. A control zone, regardless of whether it is Class C, D or E, is controlled airspace extending upward from the surface; exact dimensions are published.

Authority: TC AIM RAC - Control Zones

112. An RPAS operating manual should identify the effects of foreseeable environmental conditions because:

A. It is needed only for insurance

B. It determines airport fees

C. It replaces weather briefing

D. Weather can affect aircraft, C2 link, sensors, batteries and pilot performance

Answer: D. TP 15263 and Standard 922 require pilots/operators to understand environmental limitations and performance effects.

Authority: TP 15263 / Standard 922

113. In Southern Domestic Airspace, Class B airspace is generally found:

A. From 12,500 ft ASL up to the floor of Class A where designated

B. Only below 400 ft AGL

C. Only in airport aprons

D. Only above FL600

Answer: A. TC AIM RAC describes Class B as controlled low-level airspace, generally from 12,500 ft ASL up to Class A where designated.

Authority: TC AIM RAC - Class B

114. Safety-related mandatory actions issued for an RPAS model are intended to:

A. Set resale price

B. Address identified conditions affecting continued safe operation

C. Change pilot certificate privileges

D. Replace the CFS

Answer: B. The declaration/continued-safety framework requires safety-relevant actions and information to be maintained and communicated.

Authority: CAR Part IX / Standard 922

115. A control zone described as Class D normally extends:

A. Only from 3,000 ft AGL upward

B. Only above FL180

C. From the surface to its published upper limit

D. From 400 ft AGL to 1,000 ft AGL

Answer: C. Control zones extend upward from the surface; their exact ceiling and lateral dimensions are published in aeronautical information.

Authority: TC AIM RAC - Control Zones

116. For a VFR crewed aircraft entering Class D airspace, the normal communication requirement is:

A. Obtain an IFR clearance

B. Make no radio contact

C. Call the airport manager by phone after landing

D. Establish two-way radio communication with the appropriate ATC unit before entry

Answer: D. Class D VFR entry normally requires establishing two-way communication; unlike Class C, a specific VFR clearance is not generally the entry trigger.

Authority: TC AIM RAC - Class D

117. The normal altitude ceiling for an RPAS under CAR 901.25, absent an applicable exception or authorization, is:

A. 400 ft AGL

B. 300 ft AGL

C. 500 ft AGL

D. 1,000 ft AGL

Answer: A. CAR 901.25 establishes the normal 400 ft AGL limit, with defined exceptions/authorization pathways.

Authority: CAR 901.25

118. At a controlled aerodrome, crewed aircraft must comply with:

A. Only local road rules

B. ATC clearances and instructions applicable to taxi, takeoff and landing

C. RPAS-only rules

D. No instructions when VFR

Answer: B. Controlled aerodrome operations are subject to ATC clearances/instructions under CARs and TC AIM procedures.

Authority: TC AIM RAC - Controlled Aerodromes

119. Class C airspace boundaries are shown primarily on:

A. Road maps

B. Marine charts only

C. Current VNC/VTA and other applicable aeronautical charts/publications

D. Manufacturer maps

Answer: C. TP 15263 requires Advanced pilots to identify controlled airspace from aeronautical charts.

Authority: TP 15263 Section 5 - Maps and Charts

120. Class A airspace in Canadian high-level airspace is reserved for:

A. VFR traffic

B. RPAS only

C. Gliders only

D. IFR traffic

Answer: D. Class A is controlled high-level airspace in which only IFR operations are permitted.

Authority: TC AIM RAC - Class A

121. Terminal control areas classified as Class C:

A. Have dimensions individually published; there is no single universal altitude band for every TCA

B. Always extend surface to 3,000 ft AAE

C. Always begin at 12,500 ft ASL

D. Are uncontrolled

Answer: A. Canadian controlled-airspace dimensions are designated and published; pilots must use current charts/DAH rather than assume a universal vertical range.

Authority: TC AIM RAC / DAH principles

122. An operation can require an SFOC-RPAS when:

A. The pilot prefers paper authorization

B. It falls within a category listed in CAR 903.01 or the Minister determines an SFOC is necessary for aviation safety

C. The drone battery is below 50%

D. The operation occurs on private property

Answer: B. CAR 903.01 defines SFOC-required categories and includes Ministerial determination where necessary for aviation safety.

Authority: CAR 903.01

123. RPAS operation at an advertised event is governed through:

A. A municipal noise bylaw only

B. No aviation rule

C. The applicable SFOC/event provisions of Part IX

D. A Basic certificate automatically

Answer: C. Advertised-event operations are among the special operations addressed by the SFOC framework.

Authority: CAR 903.01 / Part IX

124. The purpose of emergency/contingency procedures in an SFOC application is to:

A. Choose camera settings

B. Set pilot wages

C. Determine weather history

D. Define how abnormal and emergency situations will be managed safely

Answer: D. CAR 903.02 requires operational and risk information sufficient to assess safe conduct, including emergency procedures where applicable.

Authority: CAR 903.02

125. The period for retaining safety-assurance records is determined by:

A. The applicable CAR/Standard 922 record-retention provisions

B. The first owner's preference

C. A one-week minimum

D. The airport manager

Answer: A. Record retention is part of the regulatory declaration/verification framework and should be taken from the current CAR/Standard text.

Authority: CAR 901.194-901.201 / Standard 922

126. Some Class C or D control zones can change classification when the ATC unit closes. The pilot should determine the current status from:

A. Assumption based on local time

B. Published aeronautical information and any applicable NOTAMs

C. A social-media post

D. The drone's geofence only

Answer: B. Airspace classification may be conditional on unit operating hours; pilots must use current published information and NOTAMs.

Authority: TC AIM RAC / CFS

127. Canadian Domestic Airspace is divided into:

A. East and west only

B. Controlled and uncontrolled only

C. Northern Domestic Airspace and Southern Domestic Airspace

D. Seven FIRs only

Answer: C. TC AIM describes the NDA and SDA division, with different heading/reference conventions in northern regions.

Authority: TC AIM RAC - Canadian Domestic Airspace

128. ESCAT stands for:

A. Emergency Scheduled Control of Air Transport

B. Electronic Surveillance Control of Air Traffic

C. Emergency Safety Communications and Tracking

D. Emergency Security Control of Air Traffic

Answer: D. ESCAT is the Emergency Security Control of Air Traffic plan.

Authority: TC AIM RAC - ESCAT

129. If ESCAT restrictions are activated, a pilot should:

A. Comply with the published restrictions and obtain required ATS approval before flight

B. Ignore them below 400 ft

C. Continue VFR without contact

D. Use only a police permit

Answer: A. ESCAT can impose security controls on civil air traffic; compliance with ATS/security directions is mandatory.

Authority: TC AIM RAC - ESCAT

130. In Southern Domestic Airspace, Class A begins generally at:

A. 12,500 ft AGL

B. FL180

C. 400 ft AGL

D. FL600

Answer: B. Class A in Southern Domestic Airspace generally begins at FL180.

Authority: TC AIM RAC - Class A

131. A two-way radio failure in controlled airspace requires the pilot of a transponder-equipped crewed aircraft to:

A. Select 7500

B. Select 7700 only in every case

C. Follow lost-communications procedures and select code 7600 as applicable

D. Turn off the transponder

Answer: C. 7600 is the standard transponder code for radio communication failure; pilots must also follow the applicable lost-communications procedure.

Authority: TC AIM COM/RAC - Communications Failure

132. Knowingly removing or interfering with an aerodrome marker, marking, light or signal used for air navigation is:

A. Permitted after dark

B. Allowed for RPAS pilots

C. A provincial matter only

D. Prohibited except as authorized

Answer: D. Aerodrome markings, lights and signals are protected safety infrastructure under Canadian aviation rules.

Authority: CAR Part III / TC AIM AGA

133. In much of Northern Domestic Airspace, navigation references use:

A. True rather than magnetic directions because magnetic references become unreliable near the magnetic pole

B. Only magnetic headings

C. Grid headings in every location

D. Runway numbers only

Answer: A. TC AIM explains the use of true reference in NDA because of magnetic unreliability at high latitudes.

Authority: TC AIM RAC - Northern Domestic Airspace

134. A Class C control zone normally extends:

A. From 3,000 ft to 12,500 ft AGL only

B. From the surface to a published upper limit

C. From 400 ft AGL only

D. Only above FL180

Answer: B. All control zones extend from the surface; exact upper/lateral limits are published.

Authority: TC AIM RAC - Control Zones

135. When aeronautical radio equipment is carried and its use is required, the operator must:

A. Use it only after landing

B. Listen but never transmit

C. Maintain the required listening watch and establish/maintain communications as prescribed

D. Use a cellular phone instead

Answer: C. CAR/TC AIM radio procedures prescribe listening-watch and reporting/communication requirements in applicable airspace and MF areas.

Authority: TC AIM COM/RAC

136. Class E controlled airspace that is not a control zone:

A. Always begins at 700 ft ASL everywhere

B. Always begins at the surface

C. Exists only above FL600

D. May begin above the surface at a published floor; its exact dimensions must be verified from charts/publications

Answer: D. Class E dimensions vary by designation; pilots must use current charts rather than memorize one universal floor.

Authority: TC AIM RAC - Class E

137. The transponder code 7600 signifies:

A. Radio communications failure

B. Unlawful interference

C. General emergency

D. VFR in Canada

Answer: A. 7600 is the internationally recognized code for radio communications failure.

Authority: TC AIM COM - SSR Codes

138. Airspace class and dimensions should be confirmed using:

A. A remembered value from an old exam

B. Current DAH/chart/CFS/AIP information as applicable

C. A drone sales brochure

D. Satellite imagery alone

Answer: B. TP 15263 explicitly requires use of current aeronautical charts and publications.

Authority: TP 15263 Section 5

139. An aerodrome traffic frequency (ATF) is used primarily to:

A. Obtain IFR clearance worldwide

B. Replace emergency frequency 121.5

C. Facilitate traffic awareness and coordination at or near specified aerodromes where an MF is not in effect

D. Control all Class A traffic

Answer: C. TC AIM RAC describes ATF procedures for traffic coordination at certain uncontrolled aerodromes.

Authority: TC AIM RAC - ATF

140. At an aerodrome where an MF area is designated, pilots must:

A. Use any frequency they prefer

B. Remain radio silent

C. Only call after landing

D. Follow the mandatory listening and reporting procedures applicable to that MF area

Answer: D. MF procedures are mandatory and are specified in CAR 602.97-602.103 and TC AIM RAC.

Authority: CAR 602.97-602.103 / TC AIM RAC

141. A NOTAM is used to distribute:

A. Time-critical aeronautical information concerning facilities, services, procedures or hazards

B. Long-term aircraft maintenance manuals

C. Pilot licensing records

D. Weather forecasts only

Answer: A. NOTAMs provide timely operational information that may affect flight safety or operations.

Authority: TC AIM MAP/RAC - NOTAM

142. The PIC's responsibility to avoid creating a collision hazard applies:

A. Only in uncontrolled airspace

B. At all times, regardless of automation or ATC service

C. Only above 400 ft AGL

D. Only to crewed aircraft

Answer: B. CAR Part IX and general rules impose continuing collision-avoidance responsibility on the RPAS pilot.

Authority: CAR 901.18 / 901.47 context

143. Within 3 NM of the centre of a DND aerodrome, an RPAS pilot needs:

A. Only Advanced certification

B. Only NAV CANADA authorization

C. Authorization from the Department of National Defence

D. No special action below 400 ft

Answer: C. CAR 901.47(4) specifically requires DND authorization for RPAS operations within 3 NM of a DND aerodrome.

Authority: CAR 901.47(4)

144. A declared RPAS model used for an Advanced operation must:

A. Be painted a specific colour

B. Always weigh under 1 kg

C. Carry a crewed-aircraft transponder

D. Meet the technical requirements applicable to the intended operation under the declaration/Standard 922 framework

Answer: D. The declaration identifies the technical capabilities for which the model has been shown to comply.

Authority: CAR 901.194 / Standard 922

145. If a modification changes an RPAS so it no longer conforms to its declared technical basis, the operator should:

A. Not rely on the declaration for that capability until conformity/approval is restored

B. Ignore the change if flight seems normal

C. Continue because declarations never change

D. Only update the pilot log

Answer: A. Safety assurance depends on the configuration continuing to meet the declared technical requirements.

Authority: Standard 922 / CAR declaration framework

146. A control zone is:

A. Any airspace below 400 ft

B. Controlled airspace extending upward from the surface around an aerodrome

C. Only Class C airspace

D. A temporary NOTAM area

Answer: B. TC AIM defines control zones as controlled airspace extending upward from the surface, normally around aerodromes.

Authority: TC AIM RAC - Control Zones

147. The difference between an aerodrome and an airport is that:

A. They are always identical terms

B. Only heliports are aerodromes

C. An airport is an aerodrome for which an airport certificate is in force; aerodrome is the broader term

D. Only military fields are aerodromes

Answer: C. The Aeronautics Act/CAR framework uses aerodrome broadly and airport for a certified aerodrome.

Authority: Aeronautics Act definitions / CAR Part III

148. A pilot planning an Advanced RPAS flight near an aerodrome should check:

A. Only the weather

B. Only the drone map

C. Only property ownership

D. Airspace class, published aerodrome data, traffic procedures, NOTAMs and required authorization

Answer: D. TP 15263 requires integration of charts, CFS/AIP information, airspace, aerodrome procedures and operational hazards.

Authority: TP 15263 Sections 1, 5 and 6

149. If an ATS authorization includes conditions, the RPAS pilot must:

A. Comply with those conditions throughout the operation

B. Treat them as optional

C. Follow only manufacturer conditions

D. Comply only above 400 ft

Answer: A. Controlled-airspace authorization under CAR 901.71 is operational authority subject to the terms imposed by ATS.

Authority: CAR 901.71

150. Which source has the highest legal authority for a CAR requirement?

A. A flight-school blog

B. The current Canadian Aviation Regulations published by Justice Laws

C. An app summary

D. A forum post

Answer: B. The CARs are the legal regulation; guidance such as TC AIM explains procedures but is not itself the regulation.

Authority: Canadian Aviation Regulations

# Section 4 — Meteorology (4-3 / 4-4)

151. Which weather phenomenon normally presents the broadest combination of severe aviation hazards?

A. Light drizzle

B. Thunderstorms

C. Thin cirrus

D. A weak ridge

Answer: B. Thunderstorms can combine turbulence, lightning, hail, heavy precipitation, icing, wind shear and microbursts.

Authority: TP 15263 Section 4 - Thunderstorms

152. A gust front is:

A. The centre of a high-pressure system

B. A warm front above 10,000 ft

C. The leading edge of cool outflow spreading outward from a thunderstorm

D. A fog bank

Answer: C. Thunderstorm downdraft outflow can produce a sharp wind shift and strong gusts ahead of the storm.

Authority: TP 15263 Section 4 - Thunderstorms

153. Approximately 78% of dry air is:

A. Oxygen

B. Carbon dioxide

C. Argon

D. Nitrogen

Answer: D. Nitrogen is the dominant constituent of dry air.

Authority: TP 15263 Section 4 - Atmosphere

154. In Canadian METARs, prevailing visibility is normally reported in:

A. Statute miles

B. Nautical miles

C. Feet

D. Kilometres only

Answer: A. Canadian METAR visibility is commonly expressed in statute miles, with metric conventions used for certain runway visual range products.

Authority: TC AIM MET - METAR

155. A surface high-pressure system is generally associated with:

A. Strong persistent ascent at its centre

B. Subsiding air and low-level divergence

C. A mandatory thunderstorm

D. A warm front at its centre

Answer: B. High-pressure systems feature sinking motion and outward surface flow, generally favouring stability.

Authority: TP 15263 Section 4 - Pressure Systems

156. A microburst is especially hazardous because it can produce:

A. Only high cloud

B. Stable calm air

C. Severe localized wind shear and rapidly changing wind

D. A constant tailwind

Answer: C. Microbursts create intense downdrafts and divergent surface winds over a small area.

Authority: TP 15263 Section 4 - Thunderstorm Hazards

157. A SIGMET is intended to advise of:

A. Routine surface observations only

B. Airport opening hours

C. Pilot licensing changes

D. Significant en-route weather phenomena potentially hazardous to aircraft

Answer: D. SIGMETs provide warnings of significant meteorological phenomena hazardous to aviation.

Authority: TC AIM MET - SIGMET

158. A METAR is primarily:

A. A routine aviation weather observation for an aerodrome/station

B. A 24-hour flight plan

C. A NOTAM

D. A long-range seasonal forecast

Answer: A. METAR is the standard routine aerodrome weather report.

Authority: TC AIM MET - METAR

159. A SPECI is:

A. A six-hour forecast

B. A special weather report issued when specified significant changes occur

C. A radio licence

D. A chart amendment

Answer: B. SPECI is a special aerodrome meteorological report triggered by specified criteria.

Authority: TC AIM MET - SPECI

160. The code 'BKN' in a METAR means:

A. Clear below 10,000 ft

B. Scattered cloud, 1/8 to 2/8

C. Broken cloud, 5/8 to 7/8 sky cover

D. Overcast, 8/8

Answer: C. BKN denotes broken cloud coverage, an aviation ceiling layer when applicable.

Authority: TC AIM MET - METAR Cloud Amount

161. The code 'OVC' means:

A. Observed visibility clear

B. Overrunning convection

C. Outside visual conditions

D. Overcast

Answer: D. OVC indicates 8/8 cloud coverage.

Authority: TC AIM MET - METAR Cloud Amount

162. A ceiling is based on:

A. The lowest BKN or OVC layer, or vertical visibility into an obscuration, as applicable

B. The highest cloud layer

C. Any FEW layer

D. Only cirrus

Answer: A. Aviation ceiling is the height of the lowest broken/overcast layer or vertical visibility into an obscuration.

Authority: TC AIM MET - Ceiling

163. The dewpoint spread becomes smaller as:

A. Air becomes much drier

B. Air approaches saturation

C. Pressure altitude becomes zero

D. Wind becomes calm

Answer: B. Temperature approaching dewpoint indicates rising relative humidity and increased likelihood of cloud/fog.

Authority: TP 15263 Section 4 - Moisture

164. Radiation fog is most favoured by:

A. Strong wind and dry air

B. Hot dry afternoons

C. Clear skies, light wind, moist low-level air and nighttime surface cooling

D. Deep frontal mixing

Answer: C. Radiational cooling under light winds can cool near-surface air to saturation.

Authority: TP 15263 Section 4 - Fog

165. Advection fog forms when:

A. Cold air descends a mountain

B. A thunderstorm downdraft reaches the ground

C. Ice crystals fall from cirrus

D. Moist air moves over a colder surface and is cooled to saturation

Answer: D. Advection transports moist air over a cold surface, cooling it toward dewpoint.

Authority: TP 15263 Section 4 - Fog

166. Upslope fog/cloud can develop when:

A. Moist air is forced to rise along terrain and cools adiabatically

B. Dry air sinks

C. Pressure rises suddenly

D. A sea breeze stops

Answer: A. Orographic ascent cools air and can produce saturation/cloud.

Authority: TP 15263 Section 4 - Orographic Lift

167. A cold front normally has:

A. Warm air replacing cold air with no lifting

B. Advancing colder air undercutting warmer air

C. No wind shift

D. Only high cloud

Answer: B. Cold fronts are boundaries where colder air advances, forcing warmer air upward.

Authority: TP 15263 Section 4 - Fronts

168. Compared with a warm front, a cold front commonly produces:

A. Only gradual layered cloud

B. No precipitation

C. A narrower band of more abrupt weather and stronger wind shifts

D. No turbulence

Answer: C. Cold fronts often have steeper slopes and more abrupt weather changes.

Authority: TP 15263 Section 4 - Fronts

169. A warm front is characterized by:

A. Cold air undercutting warm air

B. Only convective cloud

C. No cloud sequence

D. Warm air advancing and overrunning retreating colder air

Answer: D. Warm frontal lifting is gradual and often produces layered cloud/precipitation.

Authority: TP 15263 Section 4 - Fronts

170. An occluded front develops when:

A. A cold front overtakes a warm front

B. Two highs merge

C. A sea breeze reverses

D. Fog lifts

Answer: A. Occlusion occurs as the faster-moving cold front catches the warm front.

Authority: TP 15263 Section 4 - Fronts

171. A TROWAL is:

A. A thunderstorm downdraft

B. A trough of warm air aloft associated with an occluded system

C. A runway wind report

D. A radar outage

Answer: B. TROWAL means trough of warm air aloft, an important feature of occluded cyclones.

Authority: TC AIM MET - GFA/Fronts

172. The GFA system divides Canada into:

A. Five regions

B. Ten regions

C. Seven forecast regions

D. Twelve regions

Answer: C. The Canadian Graphic Area Forecast program uses seven GFA regions.

Authority: TC AIM MET - GFA

173. Convection is vertical motion caused primarily by:

A. Magnetic variation

B. Runway slope

C. Satellite geometry

D. Unequal heating and buoyancy, with warm air rising

Answer: D. Surface heating can create buoyant thermals and convective cloud.

Authority: TP 15263 Section 4 - Lifting Processes

174. Orographic lift occurs when:

A. Air is forced upward by rising terrain

B. Air sinks behind a front

C. Wind stops

D. Pressure is constant

Answer: A. Terrain mechanically forces air to rise and cool.

Authority: TP 15263 Section 4 - Lifting Processes

175. TAFs are normally issued on a scheduled cycle:

A. Only once per week

B. At specified routine times, commonly every six hours for many Canadian TAF locations

C. Only after a pilot asks

D. Every 15 minutes

Answer: B. TAFs are scheduled aerodrome forecasts; issue frequency/validity is specified in Canadian meteorological services.

Authority: TC AIM MET - TAF

176. A TAF describes expected weather:

A. Across an entire FIR only

B. Only at the runway threshold

C. At and in the vicinity of a specified aerodrome for a defined validity period

D. For the next month

Answer: C. TAFs are terminal aerodrome forecasts with a defined validity period.

Authority: TC AIM MET - TAF

177. The group 'TEMPO' in a TAF indicates:

A. A permanent change

B. The end of the forecast

C. A NOTAM reference

D. Temporary fluctuations expected during the stated period

Answer: D. TEMPO identifies temporary forecast conditions expected to occur for limited portions of the period.

Authority: TC AIM MET - TAF Change Groups

178. The group 'BECMG' in a TAF indicates:

A. Conditions expected to become the stated values during the indicated period

B. A thunderstorm only

C. No change

D. A cancelled forecast

Answer: A. BECMG denotes a gradual or indeterminate-time transition within the stated period.

Authority: TC AIM MET - TAF Change Groups

179. The group 'FM' in a TAF indicates:

A. A temporary fluctuation

B. A relatively rapid change beginning at the specified time

C. Fog only

D. Freezing rain only

Answer: B. FM marks a new forecast condition beginning from a specified time.

Authority: TC AIM MET - TAF Change Groups

180. A GFA Clouds and Weather chart includes:

A. Only upper winds

B. Only NOTAMs

C. Major cloud, precipitation, visibility and weather features for the forecast region

D. Only airport diagrams

Answer: C. The GFA Clouds and Weather chart depicts significant cloud/weather conditions over the region.

Authority: TC AIM MET - GFA

181. A GFA Icing, Turbulence and Freezing Level chart includes:

A. Runway lighting

B. Airspace classes

C. Radio frequencies

D. Forecast icing/turbulence areas and freezing-level information

Answer: D. The companion GFA chart focuses on icing, turbulence and freezing levels.

Authority: TC AIM MET - GFA

182. The freezing level is:

A. The altitude at which the air temperature is 0°C

B. The altitude where all clouds stop

C. Always 10,000 ft ASL

D. The runway elevation

Answer: A. Freezing-level information is important for icing assessment.

Authority: TC AIM MET - Freezing Level

183. Structural icing requires:

A. Clear skies only

B. Visible moisture and an aircraft surface at or below freezing under suitable conditions

C. A temperature above 20°C

D. Dry desert air

Answer: B. Icing generally requires supercooled liquid water/visible moisture and a sufficiently cold surface.

Authority: TP 15263 Section 4 - Icing

184. Freezing rain is especially hazardous because:

A. It always evaporates

B. It cannot affect rotors

C. Supercooled large droplets can freeze on contact and produce rapid clear-ice accretion

D. It occurs only above FL180

Answer: C. Freezing rain can generate severe clear icing and is a major aviation hazard.

Authority: TC AIM MET - Icing/Freezing Precipitation

185. Mechanical turbulence is often strongest:

A. Over smooth water in calm air

B. Only above the tropopause

C. At the centre of a high

D. Downwind of rough terrain, buildings and obstacles in strong wind

Answer: D. Surface roughness and obstacles disrupt airflow and create eddies/turbulence.

Authority: TP 15263 Section 4 - Turbulence

186. Wind shear is:

A. A change in wind speed and/or direction over a relatively short distance

B. A constant wind

C. Only a change in pressure

D. The same as drift

Answer: A. Wind shear can be vertical or horizontal and is especially important near the ground and convection.

Authority: TP 15263 Section 4 - Wind Shear

187. A stable atmosphere tends to favour:

A. Strong thermals and towering cumulus

B. Stratiform cloud, smoother air and limited vertical mixing

C. Unlimited vertical mixing

D. Microbursts by definition

Answer: B. Stable air resists vertical displacement and favours layered cloud.

Authority: TP 15263 Section 4 - Stability

188. An unstable atmosphere tends to favour:

A. Only fog

B. No cloud

C. Vertical motion, turbulence and convective cloud

D. Only smooth laminar flow

Answer: C. Instability supports rising parcels and convection.

Authority: TP 15263 Section 4 - Stability

189. A temperature inversion is a layer where:

A. Pressure increases with height normally

B. Wind is zero

C. Dewpoint is always 0°C

D. Temperature increases with height

Answer: D. An inversion reverses the normal lapse-rate sign and usually indicates stability.

Authority: TP 15263 Section 4 - Inversions

190. A strong inversion near the surface can:

A. Trap smoke, haze and pollutants and reduce visibility

B. Guarantee unlimited visibility

C. Eliminate fog

D. Increase vertical mixing

Answer: A. Stable inversions suppress vertical mixing and can trap particulates/moisture.

Authority: TP 15263 Section 4 - Stability

191. A sea breeze normally develops because:

A. Water heats faster than land at night

B. Land heats faster than adjacent water, producing an onshore daytime pressure gradient

C. The Coriolis force stops

D. Clouds pull air inland

Answer: B. Differential daytime heating drives local onshore circulation.

Authority: TP 15263 Section 4 - Local Winds

192. A land breeze normally occurs:

A. Only at noon

B. Only in winter storms

C. At night when land cools faster than water and flow may be offshore

D. Only above 10,000 ft

Answer: C. Nighttime differential cooling can reverse the local pressure gradient.

Authority: TP 15263 Section 4 - Local Winds

193. Katabatic wind is:

A. Warm air rising a slope

B. A jet stream

C. A sea breeze

D. Cold dense air flowing downslope under gravity

Answer: D. Katabatic winds are downslope drainage flows of dense cold air.

Authority: TP 15263 Section 4 - Local Winds

194. Anabatic wind is:

A. Upslope flow produced by daytime heating of terrain

B. Downslope cold drainage

C. A frontal jet

D. A thunderstorm outflow

Answer: A. Solar heating of slopes can produce upslope anabatic flow.

Authority: TP 15263 Section 4 - Local Winds

195. The closer together isobars are on a surface chart, the stronger the:

A. Dewpoint spread

B. Pressure gradient and generally the wind

C. Cloud ceiling

D. GPS signal

Answer: B. A tighter pressure gradient generally supports stronger winds.

Authority: TP 15263 Section 4 - Pressure Systems

196. Surface winds around a low in the Northern Hemisphere tend to:

A. Spiral outward clockwise

B. Flow straight outward

C. Spiral inward counter-clockwise

D. Remain calm

Answer: C. Friction and Coriolis combine to produce counter-clockwise convergent flow around lows in the Northern Hemisphere.

Authority: TP 15263 Section 4 - Pressure Systems

197. Surface winds around a high in the Northern Hemisphere tend to:

A. Spiral inward counter-clockwise

B. Always blow north

C. Remain calm

D. Spiral outward clockwise

Answer: D. Highs are characterized by clockwise divergent flow in the Northern Hemisphere.

Authority: TP 15263 Section 4 - Pressure Systems

198. A PIREP is:

A. A pilot report of observed in-flight weather conditions

B. A terminal forecast

C. A maintenance release

D. A NOTAM

Answer: A. PIREPs provide real-time pilot observations of weather such as turbulence, icing and cloud conditions.

Authority: TC AIM MET - PIREP

199. AIRMET information addresses:

A. Runway closures only

B. Specified potentially hazardous weather of lower intensity/coverage than SIGMET criteria but operational significance

C. Aircraft registration

D. Pilot licensing

Answer: B. AIRMETs provide advisory information on specified weather hazards relevant to aviation.

Authority: TC AIM MET - AIRMET

200. Before an RPAS flight, weather assessment should include:

A. Only temperature

B. Only sunrise time

C. Current observations, forecasts, wind/gusts, visibility, cloud, precipitation, convective hazards and changes during the operation

D. Only a consumer weather icon

Answer: C. TP 15263 requires pilots to assess meteorological conditions and their operational effects.

Authority: TP 15263 Section 4 / Section 6

# Section 5 — Navigation (5-1 / 5-2)

201. Which GNSS constellation was developed by the United States?

A. Galileo

B. BeiDou

C. GPS

D. GLONASS

Answer: C. GPS is the United States Global Positioning System; other global constellations include Galileo, BeiDou and GLONASS.

Authority: TP 15263 Section 5 - GNSS

202. Compass deviation is:

A. The angle between true and magnetic north

B. Wind drift

C. Ground track

D. The error between a magnetic compass indication and magnetic heading caused by local magnetic influences

Answer: D. Deviation is compass error produced by magnetic influences associated with the vehicle/equipment.

Authority: TP 15263 Section 5 - Definitions

203. Because radio signals travel near the speed of light, a timing error of 0.001 s corresponds to a range error of roughly:

A. 300 km

B. 30 km

C. 3 km

D. 300 m

Answer: A. At about 300,000 km/s, one millisecond corresponds to roughly 300 km of propagation distance.

Authority: TP 15263 Section 5 - GNSS Principles

204. Magnetic variation is:

A. Compass installation error

B. The horizontal angle between true north and magnetic north

C. The difference between airspeed and groundspeed

D. A radio-frequency shift

Answer: B. Variation describes the geographic difference between true and magnetic north.

Authority: TP 15263 Section 5 - Definitions

205. Latitude measures angular position:

A. East or west of Greenwich

B. Above sea level

C. North or south of the equator

D. Relative to magnetic north

Answer: C. Latitude is angular distance north/south of the equator.

Authority: TP 15263 Section 5 - Definitions

206. Longitude measures angular position:

A. North or south of the equator

B. Above ground

C. Relative to a runway

D. East or west of the prime meridian

Answer: D. Longitude is angular distance east/west of the prime meridian.

Authority: TP 15263 Section 5 - Definitions

207. A track is:

A. The path of an aircraft over the surface of the Earth

B. The direction the nose points only

C. A compass error

D. A radio bearing only

Answer: A. Track describes the actual/intended path over the ground.

Authority: TP 15263 Section 5 - Definitions

208. Heading is:

A. The ground path only

B. The direction in which the aircraft's longitudinal axis/nose is pointed

C. Distance travelled

D. Wind speed

Answer: B. Heading is aircraft orientation; track may differ because of wind.

Authority: TP 15263 Section 5 - Definitions

209. Drift angle is caused primarily by:

A. Magnetic variation only

B. GPS clock error

C. Wind displacing the aircraft from the heading toward a different track

D. Altitude

Answer: C. Crosswind creates an angular difference between heading and track.

Authority: TP 15263 Section 5 - Definitions

210. Groundspeed is:

A. Always equal to indicated airspeed

B. Wind speed

C. Rotor speed

D. The aircraft's speed relative to the ground

Answer: D. Groundspeed is the rate of movement over the ground and reflects the wind effect on airspeed.

Authority: TP 15263 Section 5 - Pilot Navigation

211. A headwind generally causes groundspeed to:

A. Decrease relative to true airspeed

B. Increase

C. Remain identical

D. Become zero in every case

Answer: A. Headwind subtracts from the aircraft's progress over the ground.

Authority: TP 15263 Section 5 - Airspeed vs Groundspeed

212. A tailwind generally causes groundspeed to:

A. Decrease

B. Increase relative to true airspeed

C. Remain identical

D. Become negative

Answer: B. Tailwind adds to movement over the ground.

Authority: TP 15263 Section 5 - Airspeed vs Groundspeed

213. A VNC is designed primarily for:

A. Airport apron control

B. Only IFR high-level routes

C. Visual navigation over broader areas at a scale suitable for VFR planning

D. Weather radar

Answer: C. VNC charts support visual navigation and airspace/topographic interpretation.

Authority: TP 15263 Section 5 - Aeronautical Charts

214. A VTA provides:

A. Only oceanic routes

B. Only weather

C. Only NOTAMs

D. More detailed visual navigation information around selected terminal areas

Answer: D. VTAs are larger-scale terminal-area visual charts for complex airspace/terrain near major centres.

Authority: TP 15263 Section 5 - Aeronautical Charts

215. Contour lines on a topographic/aeronautical chart join points of:

A. Equal elevation

B. Equal pressure

C. Equal temperature

D. Equal magnetic variation

Answer: A. Contours depict terrain elevation and relief.

Authority: TP 15263 Section 5 - Maps and Charts

216. Closely spaced contour lines indicate:

A. Flat terrain

B. Steep terrain

C. Low pressure

D. Strong wind

Answer: B. Rapid elevation change over short horizontal distance produces close contours.

Authority: TP 15263 Section 5 - Maps and Charts

217. Before relying on an aeronautical chart, a pilot should verify:

A. Only its colour

B. The printer brand

C. Its edition/currency and applicable updates

D. The pilot's licence number

Answer: C. TP 15263 requires determining validity/currency of aeronautical charts.

Authority: TP 15263 Section 5 - Maps and Charts

218. The Canada Flight Supplement is used to obtain:

A. Only weather radar images

B. Only pilot licensing rules

C. Only aircraft specifications

D. Aerodrome/facility data, communications and operational information

Answer: D. The CFS is a key aeronautical publication for Canadian aerodrome/facility information.

Authority: TP 15263 Section 5 - CFS

219. UTC is used in aviation because it:

A. Provides a common time reference independent of local time zones

B. Changes with each province

C. Uses magnetic north

D. Is tied to runway headings

Answer: A. A common global time standard avoids ambiguity in aviation operations.

Authority: TP 15263 Section 5 - Time and Longitude

220. To convert Toronto EDT (UTC-4) 14:00 local to UTC, use:

A. 10:00 UTC

B. 18:00 UTC

C. 14:00 UTC

D. 22:00 UTC

Answer: B. During EDT, add four hours to local time to obtain UTC.

Authority: TP 15263 Section 5 - UTC Conversion

221. To convert Toronto EST (UTC-5) 14:00 local to UTC, use:

A. 09:00 UTC

B. 14:00 UTC

C. 19:00 UTC

D. 20:00 UTC

Answer: C. During EST, add five hours to local time to obtain UTC.

Authority: TP 15263 Section 5 - UTC Conversion

222. A bearing is:

A. Aircraft speed

B. Altitude above ground

C. Cloud amount

D. The horizontal direction from one point to another, expressed relative to a reference north

Answer: D. Bearing is a direction to/from a point.

Authority: TP 15263 Section 5 - Definitions

223. When applying variation to convert a true direction to magnetic, the pilot must:

A. Use the local variation value and its east/west sense correctly

B. Always add 10 degrees

C. Ignore variation in Canada

D. Use compass deviation instead

Answer: A. True/magnetic conversion depends on the local published variation.

Authority: TP 15263 Section 5 - Variation

224. RTK GNSS can provide:

A. Only 10-km accuracy

B. Centimetre-level positioning under suitable conditions using carrier-phase corrections

C. No altitude information

D. Only magnetic heading

Answer: B. Real-Time Kinematic methods use correction data/carrier-phase processing for high-precision positioning.

Authority: TP 15263 Section 5 - GNSS Augmentation

225. DGPS improves GNSS accuracy by:

A. Increasing aircraft power

B. Changing magnetic variation

C. Applying corrections derived from a known reference position

D. Using barometric altitude only

Answer: C. Differential corrections reduce common GNSS errors.

Authority: TP 15263 Section 5 - GNSS

226. GNSS performance can degrade because of:

A. Only high humidity

B. Only low battery temperature

C. Runway lighting

D. Poor satellite geometry, signal blockage, interference or jamming

Answer: D. TP 15263 requires pilots to understand GNSS signal loss/interference and satellite geometry.

Authority: TP 15263 Section 5 - GNSS

227. Multipath GNSS error occurs when:

A. Signals reflect from surfaces and reach the antenna by multiple paths

B. The battery is low

C. The compass is uncalibrated

D. The aircraft is stationary

Answer: A. Reflected GNSS signals can distort measured travel time and position.

Authority: TP 15263 Section 5 - GNSS

228. Loss of GNSS during an RPAS flight can affect:

A. Only camera colour

B. Position hold, navigation, geofencing and automated return functions depending on system design

C. Only registration

D. Only radio licensing

Answer: B. Pilots must understand how GNSS loss affects the aircraft's specific flight-control modes.

Authority: TP 15263 Section 5 - GNSS Loss

229. ADS-B stands for:

A. Air Data Signal Beacon

B. Automatic Drone Separation-Band

C. Automatic Dependent Surveillance-Broadcast

D. Aerodrome Direction Service-Broadcast

Answer: C. ADS-B is a surveillance technology in which aircraft broadcast position/identity derived from onboard navigation.

Authority: TP 15263 Section 5 - Other Radio/Radar Aids

230. ADS-B 'dependent' means the surveillance data depends on:

A. Ground radar only

B. Pilot voice estimates

C. Runway lights

D. Onboard navigation sources such as GNSS

Answer: D. ADS-B position reports depend on onboard navigation data.

Authority: TC AIM COM - Surveillance

231. An ATC transponder responds to:

A. Interrogations from surveillance systems by transmitting coded information

B. Weather radar only

C. GNSS satellites

D. VHF voice calls

Answer: A. A transponder supports cooperative surveillance by replying to interrogations.

Authority: TP 15263 Section 5 - ATC Transponder

232. VOR stands for:

A. Variable Output Radar

B. VHF Omnidirectional Range

C. Visual Orientation Receiver

D. Vertical Orbital Radio

Answer: B. VOR is a VHF radio navigation aid providing azimuth information.

Authority: TP 15263 Section 5 - Radio Navigation Aids

233. VHF propagation for aeronautical communications is predominantly:

A. Ground wave around the Earth indefinitely

B. Ionospheric skywave only

C. Line of sight

D. Underwater

Answer: C. VHF range is largely limited by radio horizon/line-of-sight geometry.

Authority: TP 15263 Section 5 - Radio Theory

234. Increasing antenna height generally:

A. Always reduces range

B. Has no effect

C. Changes magnetic variation

D. Increases VHF line-of-sight radio range

Answer: D. Higher antennas increase the radio horizon.

Authority: TP 15263 Section 5 - Radio Theory

235. Buildings, terrain and the aircraft itself can affect RF links by:

A. Blocking, reflecting or attenuating signals

B. Changing UTC

C. Changing true north

D. Creating pressure altitude

Answer: A. RF propagation can be impaired by obstruction, shadowing and multipath.

Authority: TP 15263 Section 5 - RF Environment

236. An RF spectrum analyzer can help an RPAS operator:

A. Measure runway slope

B. Identify signal activity/interference across frequencies

C. Calculate dewpoint

D. Determine magnetic variation

Answer: B. TP 15263 explicitly includes assessing RF environments and the function of spectrum analyzers.

Authority: TP 15263 Section 5 - Radio Theory

237. Command-and-control link interference can result in:

A. Higher air density

B. Better GNSS geometry

C. Latency, packet loss, degraded control or lost-link behaviour

D. Lower wind

Answer: C. RF interference can compromise the C2 link; pilots must understand system-specific lost-link behaviour.

Authority: TP 15263 Sections 5 and 6

238. A flight planning area should be rejected or modified if:

A. It lacks a scenic view

B. The pilot has extra batteries

C. The drone has GPS

D. Airspace, obstacles, people, property/access or RF/weather risks cannot be acceptably controlled

Answer: D. TP 15263 requires evaluation of airspace, site access, hazards, weather and operational constraints.

Authority: TP 15263 Sections 5 and 6

239. The distance between two latitude points is useful because one minute of latitude is approximately:

A. One nautical mile

B. One statute mile exactly

C. Ten nautical miles

D. One kilometre

Answer: A. The nautical mile is historically tied to one minute of latitude.

Authority: TP 15263 Section 5 - Scale and Units

240. One nautical mile equals approximately:

A. 1.000 km

B. 1.852 km

C. 1.609 km

D. 2.500 km

Answer: B. The international nautical mile is exactly 1,852 metres.

Authority: TP 15263 Section 5 - Units

241. One statute mile equals approximately:

A. 1.852 km

B. 2.000 km

C. 1.609 km

D. 0.852 km

Answer: C. One statute mile is approximately 1.609 kilometres.

Authority: TP 15263 Section 5 - Units

242. A chart scale of 1:500,000 means:

A. 500,000 cm on chart equals 1 cm ground

B. One inch equals 500 km exactly

C. Scale is irrelevant

D. One unit on the chart represents 500,000 of the same units on the ground

Answer: D. Representative fraction scales compare identical units.

Authority: TP 15263 Section 5 - Scale

243. When planning a route near controlled airspace, the pilot should:

A. Plot the operating area against current charted boundaries and verify required ATS authorization

B. Rely only on the drone's map

C. Assume 400 ft AGL is always uncontrolled

D. Ignore vertical boundaries

Answer: A. Advanced pilots must identify airspace class/proximity and comply with CAR 901.71.

Authority: TP 15263 Section 5 / CAR 901.71

244. Magnetic heading differs from true heading primarily because of:

A. Wind only

B. Magnetic variation, with compass deviation additionally affecting compass indication

C. Groundspeed

D. Altitude

Answer: B. Variation converts true/magnetic references; deviation affects the compass itself.

Authority: TP 15263 Section 5 - Variation/Deviation

245. If a strong crosswind exists, maintaining a desired ground track generally requires:

A. Pointing directly downwind

B. Ignoring heading

C. A wind-correction angle into the wind

D. Reducing altitude only

Answer: C. A crab/wind-correction angle counters lateral drift.

Authority: TP 15263 Section 5 - Pilot Navigation

246. For RPAS range/endurance planning, a strong outbound tailwind can be deceptive because:

A. Tailwind always reduces battery use both ways

B. Wind has no effect on multirotors

C. GNSS cancels wind

D. The return leg may face a strong headwind and require much more time/energy

Answer: D. TP 15263 explicitly requires considering wind effects on range and endurance.

Authority: TP 15263 Section 5 - Flight Planning

247. A GNSS receiver needs multiple satellites primarily to:

A. Solve position and receiver clock error through ranging geometry

B. Determine battery capacity

C. Measure air pressure

D. Generate radio licences

Answer: A. GNSS positioning uses pseudorange measurements from multiple satellites to solve receiver position/time.

Authority: TP 15263 Section 5 - GNSS Principles

248. Poor GNSS satellite geometry is commonly described by:

A. Low dewpoint spread

B. High dilution of precision (DOP)

C. High QNH

D. Low magnetic variation

Answer: B. Widely separated satellites generally provide better geometry; clustered geometry increases DOP/error sensitivity.

Authority: TP 15263 Section 5 - GNSS Performance

249. Before relying on an automated return-to-home function, the pilot should verify:

A. Only camera mode

B. Only SD card capacity

C. Home point, return altitude/route hazards, GNSS/C2 assumptions and manufacturer behaviour

D. Only propeller colour

Answer: C. Flight planning must account for system-specific automation and lost-link/RTH behaviour.

Authority: TP 15263 Sections 2, 5 and 6

250. When using a GNSS-derived map display near an airspace boundary, the pilot should:

A. Treat the display as the sole legal source of airspace status

B. Assume the geofence exactly matches regulated airspace

C. Ignore vertical limits if below 400 ft AGL

D. Cross-check position and airspace against current authoritative aeronautical information and account for navigation uncertainty

Answer: D. TP 15263 requires pilots to use current aeronautical charts/publications and understand GNSS limitations; consumer or onboard mapping does not replace authoritative airspace information.

Authority: TP 15263 Section 5 - Maps, Charts and GNSS

# Section 6 — Flight Operations (6-1 / 6-2)

251. Which CAR section governs the information/authorization basis for Advanced RPAS operation in controlled airspace?

A. CAR 602.96

B. CAR 903.04 only

C. CAR 101.01 only

D. CAR 901.71

Answer: D. CAR 901.71 specifically addresses operation in controlled airspace under the Advanced division.

Authority: CAR 901.71

252. The part of an aerodrome used for parking, loading/unloading, refuelling and servicing aircraft is the:

A. Apron

B. Runway

C. Taxiway

D. Control zone

Answer: A. The apron is the area intended for accommodation of aircraft for loading, refuelling, parking or maintenance.

Authority: TC AIM AGA - Aerodrome Terminology

253. An ATS authorization may specify how two-way communications will be maintained. The RPAS pilot must:

A. Switch to text messaging without approval

B. Use the authorized method and remain available as required

C. Use any consumer app

D. Ignore communications after takeoff

Answer: B. CAR 901.71 authorizations can include operational conditions, including communications.

Authority: CAR 901.71

254. The collective term for the manoeuvring area plus apron is:

A. Control zone

B. Runway strip

C. Movement area

D. Terminal area

Answer: C. Movement area includes the manoeuvring area and apron(s).

Authority: TC AIM AGA - Definitions

255. Within an MF area, the operator of a radio-equipped aircraft must:

A. Remain radio silent

B. Use 121.5 only

C. Use cellular voice only

D. Maintain the prescribed listening watch and make required reports

Answer: D. MF procedures are mandatory under CAR 602.97-602.103 and TC AIM RAC.

Authority: CAR 602.97-602.103 / TC AIM RAC

256. An Advanced RPAS pilot conducting a site survey should identify:

A. Airspace, aerodromes, obstacles, people, property/access, weather, RF environment and emergency considerations

B. Only battery count

C. Only camera direction

D. Only sunrise

Answer: A. TP 15263 flight operations and Part IX site-planning duties require systematic hazard identification.

Authority: TP 15263 Section 6 / CAR Part IX

257. A pre-flight inspection should confirm:

A. Only the camera lens

B. Aircraft serviceability, configuration, batteries, propellers/structure, firmware/settings and required equipment

C. Only registration marks

D. Only weather

Answer: B. TP 15263 requires pilots to assess aircraft serviceability/configuration before operation.

Authority: TP 15263 Sections 2 and 6

258. If a checklist is interrupted, best practice is to:

A. Continue from memory

B. Skip the remaining items

C. Return to a known point or restart the checklist to ensure no item was missed

D. Assume automation completed it

Answer: C. TP 15263 highlights correct checklist use and managing interruptions to prevent omissions.

Authority: TP 15263 Human Factors / Operations

259. Automation complacency is hazardous because:

A. Automation makes aircraft illegal

B. Automation always fails

C. It increases air density

D. The pilot may stop actively monitoring system state and miss mode/errors

Answer: D. TP 15263 specifically addresses automation and complacency as human-factors risks.

Authority: TP 15263 - Human Factors

260. Positive transfer of control between RPAS crew members should include:

A. An explicit verbal handoff and acknowledgement identifying who has control

B. Silent assumption

C. Only eye contact

D. A text after landing

Answer: A. TP 15263 Section 8 requires clear verbal elements for positive transfer of control/visual following.

Authority: TP 15263 Section 8 - Crew Communications

261. A visual observer's primary role is to:

A. Replace the PIC

B. Assist the pilot in maintaining situational awareness/visual observation and reporting hazards as assigned

C. Issue ATC clearances

D. Perform maintenance during flight

Answer: B. Visual observers support the PIC but do not remove the PIC's responsibility.

Authority: TP 15263 / CAR Part IX

262. A flyaway contingency should include:

A. Only buying a new drone

B. Turning off all radios

C. Predetermined actions, emergency contacts, airspace/ATS notification criteria and tracking/recovery considerations

D. Ignoring the event

Answer: C. TP 15263 Section 6 requires emergency procedures and typical contacts for flyaways and other site emergencies.

Authority: TP 15263 Section 6 - Emergency Procedures

263. If control-link quality degrades unexpectedly, the pilot should:

A. Continue farther away

B. Disable return-to-home

C. Ignore warning messages

D. Apply the manufacturer's abnormal/lost-link procedures before control is lost if practicable

Answer: D. C2-link limitations and lost-link behaviour are core knowledge areas in TP 15263.

Authority: TP 15263 Sections 5 and 6

264. Battery performance is typically degraded by:

A. Cold temperature and high power demand

B. A current chart

C. A good GNSS fix

D. Lower payload

Answer: A. Temperature and load affect battery capacity, voltage sag and endurance.

Authority: TP 15263 Sections 2, 4 and 6

265. A safe charging area for lithium batteries should:

A. Be beside flammable liquids

B. Follow manufacturer instructions, reduce fire exposure, provide supervision and appropriate fire/emergency provisions

C. Be unattended on carpet

D. Ignore damaged batteries

Answer: B. TP 15263 occupational safety includes charging areas and fire-safety equipment.

Authority: TP 15263 Section 6 - Occupational Safety

266. A swollen, punctured or otherwise damaged lithium battery should:

A. Be fast-charged

B. Be flown until empty

C. Be removed from service and handled/disposed of according to manufacturer and safety guidance

D. Be stored in a hot vehicle

Answer: C. Damaged lithium batteries present fire/thermal-runaway hazards.

Authority: TP 15263 Sections 2 and 6

267. A launch/recovery area should be selected to:

A. Maximize pedestrian traffic

B. Be under overhead wires

C. Be next to loose gravel in propwash

D. Control access, avoid obstacles/debris and provide adequate margins for expected aircraft behaviour

Answer: D. TP 15263 Section 6 requires assessment of launch/recovery points, obstacles and access.

Authority: TP 15263 Section 6

268. Loose objects near a multirotor launch point are hazardous because:

A. Rotor wash can turn them into projectiles or contaminate propulsion/sensors

B. They improve lift

C. They strengthen GNSS

D. They reduce noise

Answer: A. Foreign-object/debris hazards are a basic launch-site safety concern.

Authority: TP 15263 Section 6 - Launch/Recovery

269. The primary reason to brief crew before flight is to:

A. Meet a photography requirement

B. Create a shared mental model of roles, communications, hazards, limits and contingencies

C. Replace checklists

D. Avoid weather review

Answer: B. Crew coordination and communication are explicit Advanced knowledge requirements.

Authority: TP 15263 Sections 3, 6 and 8

270. Post-flight actions should include:

A. Deleting all logs

B. Charging damaged batteries immediately

C. Securing the aircraft, identifying damage/anomalies, recording required information and addressing maintenance issues

D. Ignoring minor strikes

Answer: C. TP 15263 includes post-flight actions and maintenance/serviceability follow-up.

Authority: TP 15263 Section 6

271. A runway with magnetic direction 276° is normally designated:

A. Runway 27

B. Runway 276

C. Runway 280

D. Runway 28

Answer: D. Runway numbers use magnetic direction rounded to the nearest 10 degrees with the final zero omitted.

Authority: TC AIM AGA - Runway Designation

272. A standard rectangular traffic circuit normally contains:

A. Crosswind, downwind, base and final legs after departure/upwind

B. Only final

C. Only downwind

D. A circular orbit

Answer: A. TC AIM RAC describes the standard aerodrome traffic circuit pattern.

Authority: TC AIM RAC - Traffic Circuit

273. Red steady or flashing obstruction lights are commonly used to mark:

A. Taxiway centre lines

B. Tall structures/obstacles that may be hazardous to aviation

C. Class C boundaries

D. Wind direction

Answer: B. Aeronautical obstruction marking/lighting identifies objects that may affect flight safety.

Authority: TC AIM AGA - Obstruction Marking/Lighting

274. The purpose of an aerodrome traffic circuit is to:

A. Increase aircraft maximum speed

B. Define controlled airspace

C. Organize arriving and departing traffic into a predictable flow

D. Replace ATC

Answer: C. Predictable circuit procedures improve sequencing and situational awareness.

Authority: TC AIM RAC - Aerodrome Traffic Circuit

275. At an uncontrolled aerodrome, an RPAS pilot planning nearby operations should:

A. Assume aircraft remain above 400 ft

B. Fly over the runway if below 100 ft

C. Ignore the CFS

D. Understand the published circuit, MF/ATF procedures and likely crewed-aircraft traffic paths

Answer: D. CAR 901.47 and TP 15263 require avoiding interference with aerodrome traffic and understanding local procedures.

Authority: CAR 901.47 / TC AIM RAC

276. When wind direction favours a particular runway, crewed aircraft generally use:

A. The runway providing the most favourable headwind/crosswind conditions subject to local procedures and ATC

B. Always runway 01

C. The longest runway regardless of wind

D. Any runway chosen by RPAS traffic

Answer: A. Runway selection considers wind, operational limitations and local/ATC procedures.

Authority: TC AIM RAC / AGA

277. A windsock primarily indicates:

A. Pressure altitude

B. Surface wind direction and an approximate indication of speed

C. Cloud ceiling

D. Runway length

Answer: B. Wind direction indicators provide pilots with local surface wind information.

Authority: TC AIM AGA - Wind Direction Indicators

278. Taxiways are normally used to:

A. Store fuel

B. Define Class D

C. Connect runways with aprons and other aerodrome areas for aircraft ground movement

D. Measure visibility

Answer: C. Taxiways are defined paths for aircraft taxiing between parts of an aerodrome.

Authority: TC AIM AGA - Taxiways

279. The manoeuvring area consists of:

A. Aprons only

B. Parking lots

C. All property inside the airport fence

D. Runways and taxiways used for takeoff, landing and taxiing, excluding aprons

Answer: D. The manoeuvring area excludes aprons and includes parts used for takeoff, landing and taxiing.

Authority: TC AIM AGA - Definitions

280. A movement area incursion risk near an aerodrome is controlled by:

A. Staying outside protected operating areas unless authorized and coordinating as required

B. Relying on propeller noise

C. Flying lower

D. Using a brighter strobe

Answer: A. Aerodrome movement areas are safety-critical and subject to access/ATC/operator controls.

Authority: TC AIM AGA/RAC

281. If an RPAS operation could interfere with aircraft in an established traffic pattern, the pilot must:

A. Continue below 400 ft

B. Not conduct it in that manner

C. Ask the observer to watch

D. Use ADS-B only

Answer: B. CAR 901.47(1) prohibits operation that could interfere with aircraft operating in an established traffic pattern.

Authority: CAR 901.47(1)

282. Before a flight in controlled airspace, authorization should be checked for:

A. Only approval number

B. Only aircraft colour

C. Geographic boundaries, altitude, time window, communications and any other conditions

D. Only pilot name

Answer: C. ATS authorization conditions define the permitted operation and must be followed.

Authority: CAR 901.71

283. A site survey should identify nearby heliports because:

A. Heliports are not aerodromes

B. Helicopters cannot fly below 400 ft

C. They only matter at night

D. Helicopter traffic can operate at low altitude and may approach/depart on varied paths

Answer: D. Advanced pilots must assess nearby aerodromes/heliports and traffic hazards.

Authority: TP 15263 / CAR 901.47

284. For an operation near uninvolved people, the pilot must:

A. Apply the applicable Part IX separation/overflight rule and the RPAS declaration capability for the intended operation

B. Use 30 m for every possible operation

C. Ignore people if below 250 g

D. Use a visual observer instead of compliance

Answer: A. People-proximity privileges depend on the current regulatory category and technical declaration, not a single universal number.

Authority: CAR Part IX / Standard 922

285. An emergency landing area should:

A. Be chosen only after battery reaches 0%

B. Be considered before launch and remain suitable for foreseeable contingencies

C. Always be a road

D. Always be the original launch point

Answer: B. Pre-planning emergency/contingency locations is part of sound flight risk management.

Authority: TP 15263 Section 6

286. If a crew member reports a crewed aircraft conflict, the RPAS PIC should:

A. Continue mission until ATC calls

B. Climb above the aircraft

C. Immediately prioritize collision avoidance and take the action required to keep clear

D. Ignore the observer

Answer: C. Collision avoidance takes priority over mission objectives.

Authority: CAR Part IX / TP 15263

287. A low-battery return-to-home setting should be evaluated against:

A. Only camera mode

B. Only GPS satellite count

C. Only ambient light

D. Wind, distance, obstacles, required reserve and actual system behaviour

Answer: D. Battery/endurance and automated RTH must be assessed together with weather/site hazards.

Authority: TP 15263 Sections 2, 5 and 6

288. A strong headwind on the return leg increases the risk of:

A. Insufficient energy to reach the recovery point

B. Excess GNSS satellites

C. Higher air density

D. Reduced C2 latency

Answer: A. Wind can substantially change multirotor power demand and groundspeed, affecting endurance.

Authority: TP 15263 Section 5 - Wind on Range/Endurance

289. When flying near tall structures, turbulence can be:

A. Eliminated by the structure

B. Stronger and more erratic on the downwind side because of mechanical disturbance

C. Always smooth below rooftop height

D. Independent of wind

Answer: B. Mechanical turbulence and rotor/eddy effects develop downwind of obstacles.

Authority: TP 15263 Section 4 / Section 6

290. A pilot should not launch if:

A. The battery is 100%

B. The chart is current

C. A required safety-critical system is unserviceable or a limitation cannot be met

D. The crew briefing is complete

Answer: C. Aircraft serviceability and compliance with limitations are preconditions to safe operation.

Authority: TP 15263 Sections 2 and 6

291. Manufacturer operating limitations are:

A. Optional suggestions only

B. Superseded by a Basic certificate

C. Relevant only to warranty

D. Operational constraints the pilot must understand and follow as applicable

Answer: D. CAR 901.31 and the safety-assurance framework rely on operation in accordance with applicable manuals/limitations.

Authority: CAR 901.31

292. If weather deteriorates below the pilot's planned operating minima, the appropriate action is to:

A. Modify, delay or terminate the operation

B. Continue because the legal altitude is unchanged

C. Disable warnings

D. Rely on obstacle sensors

Answer: A. Operational decision-making requires applying conservative weather limits and stopping when risk controls are no longer effective.

Authority: TP 15263 Sections 4 and 6

293. A first-aid kit and suitable fire extinguisher are examples of:

A. Navigation aids

B. Site safety equipment identified during operational planning

C. Airspace authorizations

D. Radio licences

Answer: B. TP 15263 occupational safety asks pilots to identify safety equipment appropriate to the operation.

Authority: TP 15263 Section 6 - Occupational Safety

294. Legal site access should be confirmed because:

A. CAR 901.71 grants land access

B. An Advanced certificate grants access everywhere

C. Aviation authorization does not itself grant property access or override trespass/land-use restrictions

D. Property law never applies to RPAS

Answer: C. TP 15263 specifically identifies trespass, permissions and site access as operational planning issues.

Authority: TP 15263 Section 6 - Site Access

295. The PIC remains responsible for the safe conduct of the RPAS flight even when:

A. The aircraft is under 25 kg

B. ATC has issued authorization

C. The drone has obstacle avoidance

D. Using automation or assistance from visual observers/crew

Answer: D. Automation and supporting crew do not transfer PIC responsibility.

Authority: CAR Part IX / TP 15263

296. A contingency for GNSS loss should be based on:

A. The specific aircraft's documented mode changes, control characteristics and recovery procedures

B. A generic assumption that it will hover

C. Turning off the controller

D. Only using a compass app

Answer: A. TP 15263 requires understanding GPS loss and aircraft-system response.

Authority: TP 15263 Sections 2 and 5

297. A contingency for C2 lost link should be tested/understood before flight because:

A. All RPAS behave identically

B. The aircraft may execute programmed actions that could create new airspace or obstacle hazards

C. Lost link cannot occur

D. ATC takes control automatically

Answer: B. System-specific lost-link behaviour is a core Advanced operations knowledge area.

Authority: TP 15263 Section 6 - C2/Lost Link

298. The safest response to an unexpected person entering the operational area is to:

A. Ignore them

B. Fly over them immediately

C. Apply the pre-briefed procedure to maintain required separation, including repositioning/landing if necessary

D. Increase speed

Answer: C. People-proximity limits must be maintained throughout the operation, with contingencies for incursions.

Authority: CAR Part IX / TP 15263

299. After a hard landing or propeller strike, the aircraft should:

A. Be flown immediately to test it

B. Have only the camera checked

C. Be reset and ignored

D. Be inspected and not returned to service until its condition is acceptable under manufacturer guidance

Answer: D. Structural/propulsion damage must be identified and addressed before further flight.

Authority: TP 15263 Section 2 - Airframe Handling/Maintenance

300. If the RPAS pilot cannot maintain the conditions of an ATS authorization during a controlled-airspace operation, the pilot should:

A. Take appropriate action to remain compliant, including modifying or terminating the operation and coordinating with ATS as required

B. Continue because authorization was already issued

C. Climb above the authorized altitude

D. Switch off the radio

Answer: A. CAR 901.71 authorization is conditional authority. If its conditions can no longer be met, the operation must be brought back into compliance rather than continued outside the authorization.

Authority: CAR 901.71

# Section 8 — Radiotelephony (8-1 / 8-2)

301. The Restricted Operator Certificate-Aeronautical (ROC-A) authorizes a holder to:

A. Operate radio equipment in the aeronautical service within the certificate's scope

B. Issue ATC clearances

C. Pilot any aircraft

D. Use any radio frequency

Answer: A. ISED identifies ROC-A as the operator certificate for aeronautical-band radios used by pilots, drone operators and aerodrome ground personnel.

Authority: ISED ROC-A Overview

302. In standard radiotelephony, 'OUT' means:

A. I have received your last transmission

B. This exchange is ended and no response is expected

C. Repeat your message

D. Stand by

Answer: B. RIC-21 defines standard procedural words used in aeronautical communications.

Authority: RIC-21 - Procedural Words

303. The international distress signal with highest priority is:

A. PAN PAN

B. ROGER

C. MAYDAY

D. WILCO

Answer: C. MAYDAY identifies distress: grave and imminent danger requiring immediate assistance.

Authority: RIC-21 - Distress Communications

304. The urgency signal is:

A. MAYDAY

B. SEELONCE MAYDAY

C. ROGER

D. PAN PAN

Answer: D. PAN PAN identifies an urgent condition concerning safety without the immediate grave danger associated with distress.

Authority: RIC-21 - Urgency Communications

305. A ROC-A holder should treat the certificate as:

A. A personal radio operator qualification; current ISED requirements should be followed for issue/reprint and use

B. An ATC licence

C. An aircraft registration

D. A drone pilot certificate

Answer: A. ROC-A is a radio operator certificate distinct from an RPAS pilot certificate.

Authority: ISED ROC-A

306. When broadcasting an urgency message to all stations, the address is:

A. ANYONE LISTENING

B. ALL STATIONS, repeated as prescribed

C. EMERGENCY CENTRE

D. CONTROL ONLY

Answer: B. RIC-21 provides standard addressing/order for urgency and distress transmissions.

Authority: RIC-21 - Urgency Message Format

307. The procedural word 'ROGER' means:

A. Yes

B. I will comply

C. I have received all of your last transmission

D. End of transmission

Answer: C. ROGER acknowledges receipt; it does not mean 'yes' or 'will comply'.

Authority: RIC-21 - Procedural Words

308. 'WILCO' means:

A. I have received only

B. No

C. Repeat

D. I understand your message and will comply with it

Answer: D. WILCO combines understanding with intent to comply.

Authority: RIC-21 - Procedural Words

309. 'AFFIRM' means:

A. Yes

B. No

C. Repeat

D. Wait

Answer: A. AFFIRM is the standard affirmative response in aeronautical phraseology.

Authority: RIC-21 - Procedural Words

310. 'NEGATIVE' means:

A. Yes

B. No, or permission not granted/that is not correct

C. Message received

D. End of conversation

Answer: B. NEGATIVE is used for denial or correction.

Authority: RIC-21 - Procedural Words

311. 'SAY AGAIN' means:

A. Speak louder

B. Cancel clearance

C. Repeat all or the specified part of your last transmission

D. End transmission

Answer: C. SAY AGAIN requests repetition.

Authority: RIC-21 - Procedural Words

312. 'STANDBY' means:

A. End of conversation

B. Proceed immediately

C. Repeat

D. Wait and I will call you

Answer: D. STANDBY tells the other station to wait for a later call.

Authority: RIC-21 - Procedural Words

313. 'CORRECTION' is used when:

A. An error has been made in the transmission and corrected information follows

B. The frequency is wrong

C. A distress exists

D. The message is complete

Answer: A. Standard phraseology uses CORRECTION to repair a transmission error.

Authority: RIC-21 - Procedural Words

314. 'READ BACK' is an instruction to:

A. Transmit more slowly

B. Repeat specified information back for verification

C. End the call

D. Change frequency

Answer: B. Readback verifies safety-critical instructions/information.

Authority: RIC-21 / TC AIM COM - Readback

315. Good radio technique uses:

A. Shouting rapidly

B. Long conversational wording

C. Clear, concise speech at a steady rate with the microphone close enough for intelligibility

D. Unnecessary filler words

Answer: C. RIC-21 emphasizes clear articulation, normal tone and an appropriate speaking rate.

Authority: RIC-21 - Speech Transmission Techniques

316. Before transmitting on a frequency, an operator should:

A. Key the microphone immediately

B. Transmit a long test message

C. Use emergency priority

D. Listen to avoid interrupting another transmission

Answer: D. Good radio discipline requires listening first and avoiding interference.

Authority: RIC-21 - Operating Procedures

317. The phonetic word for the letter A is:

A. ALFA

B. ABLE

C. APPLE

D. ATLANTIC

Answer: A. ICAO spelling alphabet uses ALFA.

Authority: RIC-21 - Phonetic Alphabet

318. The phonetic word for B is:

A. BAKER

B. BRAVO

C. BETA

D. BIRD

Answer: B. ICAO spelling alphabet uses BRAVO.

Authority: RIC-21 - Phonetic Alphabet

319. The phonetic word for C is:

A. CHARLES

B. CANADA

C. CHARLIE

D. COBRA

Answer: C. ICAO spelling alphabet uses CHARLIE.

Authority: RIC-21 - Phonetic Alphabet

320. The phonetic word for D is:

A. DOG

B. DOVER

C. DRONE

D. DELTA

Answer: D. ICAO spelling alphabet uses DELTA.

Authority: RIC-21 - Phonetic Alphabet

321. The phonetic word for E is:

A. ECHO

B. EASY

C. EAGLE

D. EURO

Answer: A. ICAO spelling alphabet uses ECHO.

Authority: RIC-21 - Phonetic Alphabet

322. The phonetic word for F is:

A. FOX

B. FOXTROT

C. FRANCE

D. FALCON

Answer: B. ICAO spelling alphabet uses FOXTROT.

Authority: RIC-21 - Phonetic Alphabet

323. The phonetic word for G is:

A. GEORGE

B. GAMMA

C. GOLF

D. GREEN

Answer: C. ICAO spelling alphabet uses GOLF.

Authority: RIC-21 - Phonetic Alphabet

324. The phonetic word for H is:

A. HOW

B. HENRY

C. HARBOR

D. HOTEL

Answer: D. ICAO spelling alphabet uses HOTEL.

Authority: RIC-21 - Phonetic Alphabet

325. The phonetic word for I is:

A. INDIA

B. ITEM

C. ITALY

D. INDIGO

Answer: A. ICAO spelling alphabet uses INDIA.

Authority: RIC-21 - Phonetic Alphabet

326. The phonetic word for J is:

A. JIG

B. JULIETT

C. JAPAN

D. JUPITER

Answer: B. ICAO spelling alphabet uses JULIETT.

Authority: RIC-21 - Phonetic Alphabet

327. The phonetic word for K is:

A. KING

B. KENT

C. KILO

D. KITE

Answer: C. ICAO spelling alphabet uses KILO.

Authority: RIC-21 - Phonetic Alphabet

328. The phonetic word for L is:

A. LOVE

B. LONDON

C. LION

D. LIMA

Answer: D. ICAO spelling alphabet uses LIMA.

Authority: RIC-21 - Phonetic Alphabet

329. The phonetic word for M is:

A. MIKE

B. MARY

C. MONTREAL

D. METRO

Answer: A. ICAO spelling alphabet uses MIKE.

Authority: RIC-21 - Phonetic Alphabet

330. The phonetic word for N is:

A. NANCY

B. NOVEMBER

C. NORTH

D. NEPTUNE

Answer: B. ICAO spelling alphabet uses NOVEMBER.

Authority: RIC-21 - Phonetic Alphabet

331. The phonetic word for O is:

A. OBOE

B. ONTARIO

C. OSCAR

D. ORBIT

Answer: C. ICAO spelling alphabet uses OSCAR.

Authority: RIC-21 - Phonetic Alphabet

332. The phonetic word for P is:

A. PETER

B. PARIS

C. POLAR

D. PAPA

Answer: D. ICAO spelling alphabet uses PAPA.

Authority: RIC-21 - Phonetic Alphabet

333. The phonetic word for Q is:

A. QUEBEC

B. QUEEN

C. QUICK

D. QUARTZ

Answer: A. ICAO spelling alphabet uses QUEBEC.

Authority: RIC-21 - Phonetic Alphabet

334. The phonetic word for R is:

A. ROGER

B. ROMEO

C. RADAR

D. RIVER

Answer: B. ICAO spelling alphabet uses ROMEO.

Authority: RIC-21 - Phonetic Alphabet

335. The phonetic word for S is:

A. SUGAR

B. SOUTH

C. SIERRA

D. SIGNAL

Answer: C. ICAO spelling alphabet uses SIERRA.

Authority: RIC-21 - Phonetic Alphabet

336. The phonetic word for T is:

A. TARE

B. TORONTO

C. TIGER

D. TANGO

Answer: D. ICAO spelling alphabet uses TANGO.

Authority: RIC-21 - Phonetic Alphabet

337. The phonetic word for U is:

A. UNIFORM

B. UNCLE

C. UNITED

D. URBAN

Answer: A. ICAO spelling alphabet uses UNIFORM.

Authority: RIC-21 - Phonetic Alphabet

338. The phonetic word for V is:

A. VIENNA

B. VICTOR

C. VIOLET

D. VHF

Answer: B. ICAO spelling alphabet uses VICTOR.

Authority: RIC-21 - Phonetic Alphabet

339. The phonetic word for W is:

A. WILLIAM

B. WEST

C. WHISKEY

D. WATER

Answer: C. ICAO spelling alphabet uses WHISKEY.

Authority: RIC-21 - Phonetic Alphabet

340. The phonetic word for X is:

A. XENON

B. XTRA

C. XAVIER

D. X-RAY

Answer: D. ICAO spelling alphabet uses X-RAY.

Authority: RIC-21 - Phonetic Alphabet

341. The phonetic word for Y is:

A. YANKEE

B. YELLOW

C. YORK

D. YARD

Answer: A. ICAO spelling alphabet uses YANKEE.

Authority: RIC-21 - Phonetic Alphabet

342. The phonetic word for Z is:

A. ZEBRA

B. ZULU

C. ZERO

D. ZONE

Answer: B. ICAO spelling alphabet uses ZULU.

Authority: RIC-21 - Phonetic Alphabet

343. In ICAO radiotelephony, a decimal point is spoken as:

A. POINT

B. DOT

C. DECIMAL

D. STOP

Answer: C. DECIMAL is the prescribed spoken form to avoid ambiguity.

Authority: RIC-21 - Pronunciation of Numbers

344. The digit 3 is commonly pronounced in ICAO aviation speech as:

A. THREE exactly as conversational English

B. TRI

C. TERA

D. TREE

Answer: D. Standard radiotelephony pronunciation modifies some digits for intelligibility.

Authority: RIC-21 - Pronunciation of Numbers

345. The digit 5 is commonly pronounced as:

A. FIFE

B. FIVE

C. FIV

D. FEE

Answer: A. FIFE is the standard aviation pronunciation.

Authority: RIC-21 - Pronunciation of Numbers

346. The digit 9 is commonly pronounced as:

A. NINE

B. NINER

C. NIN

D. NOVA

Answer: B. NINER is the standard aviation pronunciation.

Authority: RIC-21 - Pronunciation of Numbers

347. A routine initial radio call should normally include:

A. Only your first name

B. A long weather report

C. Station called, your identification, position/intent or request as applicable

D. No identification

Answer: C. TP 15263 requires pilots to know the contents of a routine ATC call; RIC-21 provides standard operating procedure.

Authority: TP 15263 Section 8 / RIC-21

348. If an RPAS flyaway creates an immediate collision hazard, an emergency broadcast should:

A. Avoid giving position

B. Use only text messaging

C. Wait until the flight ends

D. Use clear priority language, identify the station/aircraft, location, altitude/direction and nature of the hazard as available

Answer: D. TP 15263 specifically requires an example of an emergency flyaway broadcast; concise hazard information supports ATS/aircraft response.

Authority: TP 15263 Section 8 - Emergencies

349. 121.5 MHz is internationally recognized primarily as:

A. The aeronautical emergency frequency

B. The universal MF frequency

C. The VFR transponder code

D. A weather radar channel

Answer: A. 121.5 MHz is the principal VHF aeronautical emergency frequency.

Authority: TC AIM COM - Emergency Frequency

350. In an MF area, the applicable frequency should be obtained from:

A. Always 121.5 MHz

B. Current CFS/chart/aeronautical publication; do not assume one frequency for every MF area

C. Always 126.7 MHz

D. Any frequency selected by the pilot

Answer: B. MF frequencies are published for each location; pilots must use current aeronautical information.

Authority: TC AIM RAC / CFS - MF Procedures

# Validation notes

This volume intentionally corrects common legacy or oversimplified training statements. Control-zone vertical limits and Class E floors are published rather than universally fixed; Advanced RPAS controlled-airspace authority is governed by CAR 901.71; DND-aerodrome proximity is addressed by CAR 901.47(4); and technical operating privileges depend on the current Part IX / Standard 922 declaration framework.

For the master quiz database, retain the Authority label and URL as structured source fields. Legal questions should be revalidated against the linked CAR section when regulations change; TC AIM procedural questions should be refreshed against the current AIM issue.

Validation build date: 4 September 2026.