# Emergency Action Plan Template

# Contact Information

## Pilot in Command

Contact Information

Ron Treleaven

(416) 317-3661

rontreleaven@gmail.com

# Flying Site Location

Address: ___________________________________________________________

GPS Coordinates: _________________N, _________________W

Nearest Intersection: __________________________________________

Directions to flying site from Intersection:

# Non-Participant Enters Flight Area

- Maintain visual contact with RPA

- Communicate the situation to the Flight Crew

- Verify

  - Check state of RPA (Status/Flight Mode)

  - Check location of RPA in flight area

- Take Actions

  - Ground Crew – Alert flight area, remove non-participant

  - Visual Observer – Monitor air traffic

  - PIC – Pause flight operation and evaluate best course of action.

- Issues

  - If non-participant is unable to be removed:

    - Stop flight operation until the issue is resolved:

  - If non-participant blocks landing location:

    - Locate suitable divert location

- Post-Incident

  - Document incident

  - Evaluate crowd control measures

# Hazardous Weather Conditions

- Maintain visual contact with RPA

- Communicate the situation to the Flight Crew

- Verify

  - Check state of RPA (Status/Flight Mode)

  - Check RPA location/altitude

- Take Actions

  - Ground Crew – Review weather forecast

  - Visual Observer – Monitor changing weather conditions

  - PIC – Evaluate if conditions exceed safe operating requirements

- Issues

  - If weather conditions are changing rapidly:

    - Maneuver the RPA to a safe divert location

    - Land immediately

  - Err on the side of caution:

    - Maneuver the RPA closer to PIC to evaluate conditions

- Post-Incident

  - Document incident

# Low Battery or Status Error

- Maintain visual contact with RPA

- Communicate the situation to the Flight Crew

- Verify

  - Check state of RPA (Status/Flight Mode)

  - Check location of RPA in flight area

- Take Actions

  - Ground Crew – Alert flight area

  - Visual Observer – Monitor air traffic and contact ATC if necessary

  - PIC – Determine safest landing location. Land as soon as possible.

- Issues

  - If disoriented:

    - Use `Return to Home' to bring RPA back for landing

    - Regain visual contact with RPA as soon as possible

  - If strong winds prevent return flight:

    - Use camera to locate suitable divert location

- Post-Incident

  - Document incident

  - Recovery telemetry logs

  - Inspect RPA and battery for physical signs of damage

# Collision with Hazard

- Maintain visual contact with RPA

- Communicate the situation to the Flight Crew

- Verify

  - Check state of RPA (Status/Flight Mode)

  - Check RPA location/altitude

- Take Actions

  - Ground Crew – Alert and clear flight area, prepare safety equipment

  - Visual Observer – Guide PIC around hazard

  - PIC – Establish positive control and land immediately

- Issues

  - If the RPA regains stability and control:

    - Maneuver the RPA to a safe divert location

    - Land immediately

  - If the RPA does not regain stability:

    - Initiate manual motor shutdown

- Post-Incident

  - Document incident

  - Inspect RPA and battery for physical signs of damage

# Fly Away/Loss of GPS

- Maintain visual contact with RPA

- Communicate the situation to the Flight Crew

- Verify

  - Check state of RPA (Status/Flight Mode)

  - Check RPA location/altitude

  - Check transmitter/tablet status and control links

- Take Actions

  - Ground Crew – Alert and clear flight area, prepare safety equipment

  - Visual Observer – Monitor air traffic and contact ATC if necessary

  - PIC – Follow RPA and attempt to re-establish connection/positive control

- Issues

  - If control is active, and RPA is drifting or moving incorrectly:

    - Lower altitude and attempt to keep RPA stationary

    - Try ‘Return to Home’ to bring RPA back for landing

    - Try switching control to ‘manual’ or attitude control

  - If control is active but RPA is not responding:

    - Try switching control to ‘manual’ or attitude control

  - If control is not active

    - Assist Ground Crew in alerting and clearing flight area

- Post-Incident

  - Document incident

  - Recovery telemetry logs

  - Inspect RPA and battery for physical signs of damage

# Lost Link

- Maintain visual contact with RPA

- Communicate the situation to the Flight Crew

- Verify

  - Check state of RPA (Status/Flight Mode)

  - Check RPA location/altitude

  - Check transmitter/tablet status and control links

- Take Actions

  - Ground Crew – Alert and clear flight area, prepare safety equipment

  - Visual Observer – Monitor air traffic and contact ATC if necessary

  - PIC – Follow RPA and attempt to re-establish connection/positive control

- Issues

  - If telemetry is active, but video feed has failed:

    - Lower altitude and attempt to keep RPA stationary

    - Try ‘Return to Home’ to bring RPA back for landing

    - Try switching control to ‘manual’ or attitude control

  - If telemetry/control is not active

    - Try restarting controller/tablet

    - Adjust antennas and attempt to improve connection while following RPA

    - Assist Ground Crew in alerting and clearing flight area

- Post-Incident

  - Document incident

  - Recovery telemetry logs

  - Inspect RPA and battery for physical signs of damage

# In-Flight or Post-Flight Fire

- Maintain visual contact with RPA

- Communicate the situation to the Flight Crew

- Verify

  - Check state of RPA (Status/Flight Mode)

  - Check RPA location/altitude

  - Check transmitter/tablet status and control links

- Take Actions

  - Ground Crew – Alert and clear flight area, prepare safety equipment

  - Visual Observer – Be prepared to call emergency services

  - PIC – Immediately terminate flight

- Issues

  - If RPA sparks a ground fire:

    - Secure site and attempt to extinguish the fire with Ground Crew

  - If unable to extinguish fire:

    - Call emergency services

    - Stop all fire suppression efforts and begin minimizing potential damage by clearing the area and removing other potential fire hazards

  - DO NOT COMPROMISE YOUR SAFETY

- Post-Incident

  - Document incident

  - Contact PI and Designated Local Authority

# Pilot Incapacitation

- Ensure the PIC is IMMEDIATELY attended to

  - Typically, the VO will be the closest person to the PIC

- Communicate the situation to the Flight Crew

- Take Actions

  - VO to pause or stop flight operation, then resume attending to PIC

  - Ground Crew must be prepared to call emergency services

  - Check transmitter/tablet status and control links

- Issues

  - Call emergency services if there is a possibility that they may be needed

  - If flight operation cannot be paused or stopped, initiate a manual termination, unless an alternative operator is available.

  - RESPONSE PRIORITIES:

    - Avoid delaying emergency services to PIC

    - Ensure RPA will not place additional persons at risk

    - Prevent the RPA from causing property damage

    - If able, land the RPA safely

- Post-Incident

  - Document incident

  - Contact PI and Designated Local Authority

# Airspace Encroachment

- Maintain visual contact with RPA

- Communicate the situation to the Flight Crew

- Verify

  - Check state of RPA (Status/Flight Mode)

  - Check location of RPA in flight area

- Take Actions

  - Ground Crew – Alert flight area, prepare for landing

  - Visual Observer – Monitor air traffic and provide

  - PIC – Evaluate best course of action

- Issues

  - If a collision is imminent:

    - PIC must take immediate evasive action

  - If a collision is a strong possibility:

    - PIC must land or terminate the flight at the nearest divert or safe location

  - If the encroachment is unsafe but not likely to result in a collision:

    - PIC must pause flight, reduce flight altitude and monitor the situation.

    - PIC may only resume flight operation when the hazard has passed

  - If the encroachment is noticeable, but not likely to cause a safety issue:

    - PIC may continue flight operations

- Post-Incident

  - Document incident

  - Review See & Avoid procedures